Niman Power

West greater Noida, Sector -12, UP (201301)

JOB SAFETY ANALYSIS

| Project: |  | Date: |  |
| --- | --- | --- | --- |
| Job Title: | Construction of cable trench | JSA no: |  |
| Location: |  | Page | 1 of |

JSA done by:

| Sr. | Name | Designation | Department | Remarks |
| --- | --- | --- | --- | --- |
| 1 |  |  | Consultant |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Contd……

| Sl. No. | Activities | Hazards | Recommended Actions / Procedures & Control Measures |
| --- | --- | --- | --- |
| 1 | Area Inspection & Test Pit | Damaging the Under Ground Utilities like HT Electrical cables /pipelines and OFC Cables. | - Prior to start of activity; area clearance shall be ensured. - Mandatory PPE’s along with guideline to be followed in the project area. - Location to be identified before any of the activity and cleared from any obstruction. - Area to be barricaded if possible. - Test pits to be done. |
| 2 | RCC or PCC breaking by Excavator or JCB Breaker | - Equipment over heat - Failure of equipment - Dust particles fall in eye. - Noise pollution (Hearing loss) - Eye injury - Musculoskeletal disorder | - Ensure the pressure withstand ratings of hoses as per standard - Check the hoses free from blockage - Check all the hoses connecting points free from loose contact and take the periodical maintenance of equipment - Barricaded the equipment area, avoid the unauthorised person entry near the equipment - Ensure the noise level within the permissible limit. |
| 3 | Excavation, Backfilling, Grading & Dumping | - Damage of underground services. - Collapse of side earth. - Fall of person into pit. - Hit by Excavator. - Fall of material into pit. - Toppling of earth Moving Equipment, Vehicle & Construction Machineries /into the Pit - Road blockage due to loose soil - Suffocation | - Work permit will be taken. - Workmen should not be within swing area of the excavator. - Sufficient working space & slope to be provided with steps and handrails. - Shoring to be done wherever required, benching. - Work area will be properly barricaded using soft barricading & excavated earth material will be kept in their specific area. - Loose soil and other materials should not be kept with 1.5 mtr from excavation pit. - Hard barricade shall be provided at the edge of excavation pit. - Safe access to be provided. - Close supervision shall be ensured. - Adequate Illumination shall be provided for night work. - Designated flag man shall be deployed for monitoring the movement of earth moving equipment. - Vehicle movement area must be demarcated. - Soil strengthening of vehicle movement area / road being done. - Excavated Soil will be loaded on Dumper after excavation - Do not keep loose soil on road - Confined space training - Gas testing to be done if required. |
| 4 | Dewatering: Dewatering pump installation and operation. | - Electric Shock. - Physical injury. - Fall of person in deep excavation Area. - Fall of materials | - Dewatering pump should be routed through RCCB and ensure body earthling. - Switch to be checked by authorized person before use. - No workmen allowed to work inside the waterlogged area while dewatering activity is in process. - Using proper PPE’S (Rubber hand gloves, Gum boot). |
| 5 | Shifting and placing of bar bending and cutting machines at bar bending and cutting yard | - Fall of machine / property damage. - Physical Injury | - Use proper lifting equipment and tools & tackles for lifting of Bar bending and cutting machine - Use tag line for controlling of uncertain movement of lifted bar bending and cutting machine. - Secure machine on the transport vehicle bed for uncertain fall. - Construct required height and level bed for bar bending and cutting machine |
| 6 | - Fixing bar bending and cutting machine. - Bar cutting & bending work - Fabricating reinforcement. | - Electrocution Trap of workman in between moveable parts. | - Proper earthling of machine (double body earth) shall be provided. - Provision of RCCB with 30mA tripping capacity. - Provide rubber mat in-front bar bending and cutting machine. - AII rotating parts of the machine shall be properly guarded - Provide switch for emergency stoppage on machine - Skilled manpower shall be deployed. - Authorized operator shall be engaged for operating the machine. - Close supervision shall be ensured. |
| 7 | Hand tying of reinforcement. | Physical injury due to sharp edge of reinforcement steel, fall of materials during handling. | - Mandatory PPE’s along with IOCL guideline to be followed in the project area. - Only skilled group shall be deployed for the job. - All the hand tools shall be checked before starting the job. - Area shall be free from any obstruction and proper housekeeping to be maintained. - Proper stand (reinforcement chair) shall be provided. |
| 8 | Shuttering & De-shuttering Work | - Improper access/egress to work location - Physical Injury - Fall of material - Hit by Object | - Provide access with steps with handrails to work location. - Use cotton Hand gloves. - Use shoulder pads, goggles. - Engage adequate nos. of workman for shifting. - Maintain coordination between working crew while fitting, shifting and placing rebar. - Use proper tools & tackles for fitting rebar. - Lifeline should be provided & used for horizontal movement. - Scaffolding to be made by only trained person. - Scaffolding inspection tag shall be by competent person. - Skilled manpower shall be deployed. - Close supervision shall be ensured. |
| 9 | Scaffolding erection & dismantling | - Fall of materials. - Fall of persons. - Collapse of scaffolding. - Physical injury. Property damage. | - Prior to start activity work permit shall be taken. - Ground level shall be compacted and levelled. - Proper job sequence shall be ensured. - Proper communication shall be ensured. - Skilled manpower shall be deployed. - Adequate support shall be ensured. - Job specific TBT (Job-Specific Toolbox Talk) shall be conducted. - Activity shall be done under close supervision. - Area to be barricaded. - Scaffolding tag shall be provided. - Proper material handling should be followed. |
| 10 | Material stacking | - Improper material stacking. - Trips, Fall & Hit Injury - Over stacking. | - Materials to be stacked properly. - Minimum height less than 2 Mtrs. to be maintained. - Barricading should be provided around the area. - Gap should be provided for easy lifting / inspection of the material. - All the personnel shall wear PPE. |
| 11 | Transportation of equipment (Excavator, crane, etc.) | - Fall of equipment/ machine. - Uncontrolled vehicle movement - Run over / Hitting people and vehicles by reversing | - Equipment /machine to be placed and tight properly on trailer bed. - Flag man to follow up the vehicle to the assigned location. - Vehicle inspection to be done. - Driver must have valid heavy license (TR-transport). - Equipment operators and drivers shall follow speed limit and traffic signs. - Ensure Mobile phone not to use. - Wheel stopper shall be provided during standing condition. |
| 12 | Loading & Unloading of excavator, crane etc. | - Toppling of equipment/ machine - Physical injury. - Property damage | - Proper ramp shall be provided. - Close supervision shall be ensured. - Area shall be barricaded. - Unauthorized entry shall be restricted. - Work permit shall be taken. - TBT (tool box talk) shall be given. |
| 13 | Hot Jobs Cutting and welding | - Leakage of inflammable gas. - Toppling of trolley. - Falling of cylinders. - Use of defective apparatus. - Welding cable damage | - Prior to start work hot work permit shall be taken. - Job specific TBT shall be given. - Trolley should be placed on firm ground and ensure prevention of toppling. - Skilled manpower shall be deployed. - Oxygen/DA cylinder is to be kept in upright position on trolley with proper tie by chain. - Cylinder must be placed on designated stand and secure by chain only. - Gas cylinder shall be stored at isolated place. - Hose coupling shall be tied of with Jubilee clamps - Flash back arrestor shall be provided. - Fire extinguisher and fire blanket shall be provided. - Mandatory PPE’s along with guideline to be followed in the project area. |

| Sr. | Name | Designation | Department | Remarks |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Reviewed and approved by

| HOD | Name | Designation | Sign |
| --- | --- | --- | --- |
| HOD user department |  |  |  |
| HOD of Concerned (Project Department) |  |  |  |
| HOD Operation Department |  |  |  |

Note:

To make JSA process effective, the defined job shall be broken down into sequential steps.  Actions / activities for each step shall be logical & linked with previous & forward action steps. All possible hazards associated with each job step shall be deliberated among team members and recorded carefully. JSA recommendations / control measures for each hazard / job step should be validated by team members & recorded in the Part-A of JSA sheet. Use additional JSA sheets as per requirements.

- Review of JSA shall be done by HOD of respective Operation Department and HOD of Concerned Maintenance Department. After review, respective HOD to put his signature with name & date.

- The Job shall be carried out with safety precautions as per JSA recommendations. HODs/ Fire & Safety & higher management personnel may also check compliance of JSA